MICROSOFT MOUSE README.DOC

This document contains information that became available

after the "Microsoft Mouse User's Guide" was printed.

MICROSOFT MOUSE HARDWARE NOTES

"Microsoft Mouse: Driver Not Installed -- Mouse Not Found"

Error Message

If you receive the message "Microsoft Mouse: Driver Not

Installed -- Mouse Not Found" after you have connected a

Serial mouse to your computer and have run MSETUP to install

the mouse software, it may indicate a serial port

configuration problem. This is particularly true when

multiple serial adapters or a serial adapter and a modem are

installed in the system. Check the configuration of each

serial port as specified in the manufacturer's installation

guide. Be sure that non-IBM serial adapters are configured

to be 100% IBM compatible. If you receive this "Microsoft

Mouse Driver Not Installed" message and cannot find any

problems with your system's serial port configuration, or you

do not have the mouse attached to a serial port, it may indicate

a mouse hardware problem and you should contact the dealer

from whom you purchased your mouse.

MICROSOFT MOUSE SOFTWARE NOTES

Removing Control Panel from Memory

If you install other Terminate and Stay Resident programs after

the Control Panel, you need to unload these programs from

memory before being able to unload Control Panel.  This is

true in general for Terminate and Stay Resident programs.

Single-drive PCs

If you have a single-drive PC without a hard disk, you will

not be able to use the Mouse Setup program (MSETUP) to

set up the mouse software.

MSETUP installs a mouse driver, MOUSE.COM. (For more

information, see "What the MSETUP Program Does" in  of the

"Microsoft Mouse User's Guide.") On a single-drive PC, you

must install the mouse driver yourself in one of two ways:

Manual Installation -- Copy MOUSE.COM onto the disk you use

to start your computer. You can then load the mouse driver

manually each time you start your system, by typing "mouse"

at the DOS prompt.

Automatic Installation -- With MOUSE.COM on the disk you use

to start your computer, you can modify your AUTOEXEC.BAT file

so that the mouse driver is loaded automatically each time you

start your system. Add the line "MOUSE" to the AUTOEXEC.BAT file

to load MOUSE.COM automatically when you start your computer.

Invoking the Control Panel In Microsoft Word

Microsoft Word recognizes the Ctrl-left mouse button and

Shift-left mouse button key combinations as Word operations.

Therefore, in order to invoke the Control Panel while

working in Word, you need to hold down the Ctrl key and then

press the Alt or Shift keys before pressing the left mouse

button.

Using the DEFAULT Expert Mouse Menu

The DEFAULT Expert Mouse Menu lets you use the mouse to

emulate keys such as the Esc, Enter, and cursor keys with

applications that don't normally support the mouse. To use

this menu, you need to load it into memory before you

start your application. To load the DEFAULT menu, type:

default

after the prompt of the drive or directory where the DEFAULT.COM

file resides.

Once the DEFAULT menu is loaded, you can perform the mouse

actions described in the following tables to emulate certain

keys in your application.

Press                     To emulate

-----                     ----------

The left mouse button     The F3 key

The right right button    A carriage return (the Return

or Enter key)

Both mouse buttons        The Esc key

Move                      To emulate

----                      ----------

The mouse to the left     The Left Arrow key

The mouse to the right    The Right Arrow key

The mouse up              The Up Arrow key

The mouse down            The Down Arrow key

To remove the DEFAULT menu from memory, type:

default off

after the prompt of the drive or directory where the DEFAULT.COM

file resides.

ADDITIONAL INFORMATION

Additional information regarding the application included in

your Microsoft Mouse package may have been included in a

readme file. Please check the application disk for a readme file.