Installing Windows 386 Display Device Drivers

To properly install the enhanced VEGA VGA device drivers provided

on this disk for Microsoft Windows 386, run the Windows 386

"SETUP" as instructed by Microsoft.

The SETUP program will ask you questions about your configuration

needs as well as what type of graphics drivers and display

adapter you wish to use.

When asked for your display adapter select "other", then insert

this disk in drive A:.

The SETUP program will then ask you to select one of the drivers

provided on this disk.  Each driver supports a different

resolution.

A variable frequency monitor will be required if you choose

either the 720 x 540 or 800 x 600 resolution drivers.

For the other two drivers provided on this disk (640 x 480 and

640 x 450) either a variable frequency or VGA compatible

monitor will be  required.

Next you will be asked to select the fonts you wish to use.  We

recommend that the VGA fonts be used for all of the drivers

provided on this disk.

Continue with the rest of the "SETUP" program to complete the

full installation of Windows 386.