DFI.SYS

The DFI.SYS driver supports both the DFI 2000 and DFI 3000 hand

scanners. DFI.SYS allows the DFI 3000 to use four revolutions,

400, 300, 200, and 100 dpi. The DFI 2000 can only provide 200 dpi

resolution.

For:	Scanner Environment	CONFIG.SYS entrees

DFI 3000 default	'device=dfi.sys'

DFI 2000 scanner	'device=dfi.sys /A'

Set up for Windows	'device=dfi.sys /W'

Double Buffer Data	'device=dfi.sys /D'

Micro Channel Adapter	'device=dfi.sys /M'

DFI 3000 Plus Scanner   'device=dfi.sys /P'

Extended memory start   'device=dfi.sys /XD000'

Reserve memory		'device=dfi.sys /K512'

While this driver was primarilly designed for use under Windows, it

works well in either Publishers or Plus, as long as there is EMS

available.

DFI.SYS will work without EMS, but in a restricted manner, mainly

the scanned image must be reduced in size. For instance, on most

640 K machines, typical scanned images sizes are:

Resolution	Image Size	DOS Memory Used

200 DPI		 4" X 6"	    120K

300 DPI		 4" X 3"	    120K

400 DPI		 4" X 1.5"	    120K

Always use a 2 color mode when scanning in images, since the scanner

only provides black and white information. This will leave more DOS

ram free to scan into.

The '/W' switch is for Windows. This enables certain LIM 4.0 calls

necessary to work under Windows. Without the '/W' switch, DFI.SYS

utilizes EMS for LIM 3.XX under Plus and Publishers.

The '/M' switch is for the DFI Micro-Channel interface.

The '/D' switch is necessary for setups that have both the DFI EMS

board and the DFI hand scanner. This is the double buffer switch, and

some EMS boards will not allow direct transfer to EMS memory with DMA,

hence double buffering is required.

The '/P' switch allows the use of the DFI 3000 Plus scanner. This

scanner has a smaller horizontal scanning area in 300 and 400 dpi.

The '/X' switch is not yet implemented, but it will allow DFI.SYS to

use extended memory. The user must give the starting address, and

of course no other extended memory drivers are allowed when using this

mode.

The '/K' switch is not yet implemented, but it will tell DFI.SYS how

much memory to reserve for scanning.

If the driver doesn't work or a bad image is scanned in, check for

interference from a printer port setup for LPT2.

Be sure that Paintbrush and the switches on the Handy Scanner are

set for the same resolution, or else there will be garbage included

to the side of your scanned image.