COMPLETE.SYS Documentation

COMPLETE.SYS is a DOS device driver that allows the Complete

Half Page Scanner to be used with PC PaintBrush. The resolution

is only 200 dpi. Dither and Brightness are set on the scanner itself.

Install the scanner and the bundled software that comes with

it. Note the software will not recognize the scanner's existence if

the scanner is actually plugged in. Run SCANCHK.EXE with the scanner

unplugged and only plug it in if you get to the part where it tells

you that its light should be on.

Options are:

/C	channel for DMA, 1 or 3 (default is channel 3)

/D	double buffering switch (for some EMS boards)

/H	hex port value, default is 3E0H (no 'H' needed)

/W	Windows 4.0 option

Examples:

device=complete.sys /H220 /W	;port = 220H ;LIM 4.0 required

Ports:

220H	260H	2A0H	2E0H	320H	360H	3A0H 	3E0H

Switch	#1	#2	#3	#4	#5	#6	#7	#8

|N.E.	|IRQ2	|IRQ3	|IRQ4	|IRQ5

3E0H	off	off	off		make sure that only one is set

3A0H	off	on 	on

360H	off	on 	off

320H	off	off	on

2E0H	on	off	off

2A0H	off	on 	on

260H	off	on 	off

220H	off	off	on