LOGITECH.SYS Documentation

LOGITECH.SYS is a DOS device driver that allows the Logitech

ScanMan to be used with PC PaintBrush. The resolutions are 400, 300,

200 and 100 dpi. Dither and Brightness are set on the scanner itself.

Port options are:	280H,	2A0H,	330H, or   340H.

Install the scanner and the bundled software that comes with

it. If all goes well, then set up LOGITECH.SYS in your CONFIG.SYS file.

Options are:

/C	channel for DMA, 1 or 3 (default is channel 3)

/D	double buffering switch (for some EMS boards)

/H	hex port value, default is 280H (no 'H' needed)

/W	Windows 4.0 option

Examples:

device=logiterch.sys /w	       ; che#ck for LIM 4.0 for Windows

device=logitech.sys /C1        ; channel 1