Programmer'S Reference  Manual

1. Introduction

The MITSUMI mouse driver enables you to use mouse hardware to move

cursor on screen and control its movement through software program.

The following manual provides information to programmers who want

to include support for MOUSE.COM  in their application

programs.  This document includes descriptions of the graphics cursor

and text cursor, and  the mouse functions.

All functions supported by MOUSE driver including:

*. All functions supported by the Microsoft mouse driver.

*. Some new function supported to MOUSE working in different

modes.

All the function call interface is compatible with Microsoft

Corp.  As it is documented in the installation and operation

manual published by Microsoft Corp.  Entitled Microsoft (r) Mouse

User's Guide for IBM Personal Computers.

2. Screen Coordinated System

The MOUSE driver is compatible with the IBM Color Graphics

Adapter, the IBM Monochrome Display Adapter, the IBM Enhanced

Graphics Adapter, and the Hercules Monochrome Graphics Adapter.

The MOUSE driver works with all of the BIOS supported screen

modes for both the IBM Color/Graphics Monitor Adapter and the IBM

Monochrome Display Adapter, these modes are:

*. High-resolution graphics mode (640 * 200, 2 colors).

*. Medium-resolution color mode (320 * 200, 4 colors).

*. Text mode -

(1) 80 column by 25 row.

(2) 40 column by 25 row.

The Hercules graphics Card is supported in monochrome graphics

mode:

*. Monochrome Graphics (720 * 348, 2 colors).

When you want to use MOUSE with the hercules high

resolution modes, follow these steps:

(1) If you are using the page at B000H, poke a 6 into the

memory location 40:49.

(2) If you are using the page at B800H, poke a 5 into 40:49.

In medium-resolution mode --

The physical coordinate is 320 pixels across by 200 pixels.

All horizontal coordinates are mapped to even coordinates in

logical coordinate system.  On the upper left-corner of the

screen is (0,0), on the bottommost right-corner of the screen

is (638,0).

In 80 column by 25 row text mode --

The logical coordinates is 640 * 200 are used.  A logical

character size of 8 * 8 pixels is assumed.  All logical

horizontal coordinates are multiples of eight, and all

logical vertical coordinate are multiples of eight.

In 40 column by 25 row text mode --

The logical coordinates is 640 * 200 are used.  A logical

character size of 16 * 8 pixels is assumed.  All logical

horizontal coordinates are multiples of sixteen, and all

logical vertical coordinates are multiples of eight.

In Hercules monochrome graphics mode --

The logical coordinates match the physical pixels.  The

pixels on the top row have coordinates of (0,0), (1,0), etc.

The pixels at the bottommost right of the screen is

(719,347).

Logical coordinates are used as both input and output to mouse

function calls.  User should be careful to provide the

correct values for the current screen mode.

3. Text Cursor and Graphics Cursor

MOUSE driver supports a hardware text cursor, a software text curosr

, and a graphics cursor.A hardware test cursor is a blinking cursor

which moves from one character to another on screen. This blinking

cursor may take the form of a block or underscore. A software text

cursor makes use of display attributes to change the visual appearance

of a character on screen. Movment is from character to character. A

graphics cursor is a shape that movess over on screen images. You can

choose any of these three cursor to use on screen, howere, only one

cursor can be displayed at a given timme. Allso, within your application

program, you can switch back and forth between cursors.

The characteristics of the cursor are determined by Functions 9

and 10.

1). Graphics Cursor

The cursor appears on screen or disppears from the screen

through the calling program. The graphics cursor is an array

of pixels (normally 16 by 16).  It is defined by two arrays

of bits called the screen mask and the cursor mask.

The screen mask has two parts to determine the shape or the

background of cursor pixels.  Cursor pixels are part of the

shape (0) or of the background (1) (it is ANDed with the

screen contents).  The cursor mask is used to determine the

shape color  and background color of the cursor pixels (it is

XORed with the result of the previous operation).

The operational behavior of these bit arrays are summarized

in the following table:

┌────────────────┬───────────────┬─────────────────────────┐

│ screen mask    │  cursor mask  │ resulting screen bit    │

├────────────────┼───────────────┼─────────────────────────┤

│      0         │       0       │           0             │

├────────────────┼───────────────┼─────────────────────────┤

│      0         │       1       │           1             │

├────────────────┼───────────────┼─────────────────────────┤

│      1         │       0       │      unchanged          │

├────────────────┼───────────────┼─────────────────────────┤

│      1         │       1       │        inverted         │

└────────────────┴───────────────┴─────────────────────────┘

Note that when a mouse function refers to the graphics cursor

location is referring to the point on the screen that lies

directly under the cursor's target area.  The target area is

the point in the cursor block that the mouse software uses to

determine the cursor coordinates.  This point is called the

hot spot of the cursor.  The upper left corner of the cursor

block is the hot spot for the default graphic cursor.

2).  Text Cursor

The text cursor is a cursor that can be used when the video

is in one of the text modes.  Tow kind of text cursor are

supported: a hardware text cursor and a software text cursor.

a. The hardware text cursor is the cursor actually placed on

the screen by the video controller itself.  It is defined

in terms of the scan lines of the character cell numbered

from 0, starting from the top scan line.  The numbers of

scan lines on a character cell depend on the actual video

controller and monitor (see the controller documentation

for details).

b. You can use this text cursor when your computer is in one

of the text modes. By changing the character attributes

beneath the curssor, the appearance of the character is

influenced on screen. The software text cursor is a character

code and a character attribute that replaces and/or modifies

the character cell on the screen where it is positioned.

The behaviors of this cursor is defined by 2 16-bit

values.  he format of the two values is the following:

Bit               Description

15                blinking (1) or

non blinking (0)

14-12             background color

11                high intensity (1) or

medium intensity (0)

10-8              foreground color

7-0               character code

The tow values are called the screen mask and the cursor

mask.  The screen mask is used to determine which of the

character attributes are preserved (it is ANDed with the

screen character and attribute).  The cursor mask is used

to determine which of the characteristics are changed by

the cursor (it is XORed with the result of the previous

operation).

3). Hardware text cursor

The cursor is available when the computer is in one of the

text modes. It consists of 8 pixels wide and 8 to 14 pixels

tall. Software allows you to use this cursor when you need.

Refer to function 10.

4. The Mouse Button

The mouse function calls can return the current state of the

mouse buttons and the number of times of a given button has been

pressed or released.  The button state is returned as a integer

value in the low order three bits are one or zero.  The following

table describes the bit status:

Bit 0: Left button state      (0 - Up, 1 - Down)

Bit 1: Right button state     (0 - Up, 1 - Down)

5. Make mouse function call

* Calling from assembly language program

To make mouse function calls, load the appropriate registers

( AX, BX, CX, DX ) with parameter values. Then execute software

interrupt 33H. The return values given by the mouse functions

will be installed in the registers.

Example:

; set cursor to location (150,100)

mov     ax,4    ; function call No.4

mov     cx,150  ; set horizontal to 150

mov     dx,100  ; set vertical to 100

int     33h     ; interrupt to mouse

It is important to note that before using INT 33H, should verify

presennce of the mouse driver. Executing an INT 33H will cause

uncertain results if the mouse driver is not loaded.

6. Mouse Function Call

Function List

-------------------

Functions                                       Function Number

----------------------------------------------------------------

Reset mouse driver                                      0

Enable mouse cursor display                             1

Disable mouse cursor display                            2

Read cursor location and button state of mouse          3

Set cursor location of mouse                            4

Read button presse state of mouse                       5

Read button release state of mouse                      6

Define horizontal (x) range of cursor                   7

Define vertical (y) range of cursor                     8

Define graphics mode cursor style                       9

Define text mode cursor style                           10

Read mouse motion number                                11

Define event handler entry location                     12

Enable light pen emulation function                     13

Disable light pen emulation function                    14

Define sensitivity(motion/pixel) of mouse               15

Disable cursoor display in special range                16

Reserved                                                17

Reserved                                                18

Define double speed threshold                           19

Swap event handler entry location                       20

Get mouse driver state storage size                     21

Save mouse driver state                                 22

Restore mouse driver state                              23

Reversed                                                24

Set CRT page number                                     29

Get CRt page number                                     30

---------------------------------------------------------------

Function Description

* Function 0

Mouse initialization

This function is used to reset mouse and the current status of

the mouse software. The calling program is able to determine the

presence of the mouse driver and serial port.

Input

AX = 0

Output

AX = mouse status(-1: installed, 0: not instaled)

BX = number of buttons(2 button mode, 3 button mode)

This functionresets the mouse driver to the following default:

Variable                                        Value

-----------------------------------------------------------------

----------

Internal cursor flag                            -1 (cursor concealed)

Graphics cursor shape                           horizontal oval

Text cursor                                     reverse video

User defined call mask                          all zeroes

Light pen emulation mode                        enabled

Vertical mouse motiom/pixel ratio               16 / 8

Horizontal mouse motion/pixel ratio             8 / 8

Vertical min/max cursor coordinates             0/current display mode Y-1

Horizontal min/max cursor coordinates           0/current display mode X-1

-----------------------------------------------------------------

----------

* Function 1

Set cursor show

This function is used to show current mouse position on the

screen.

Input

AX = 1

Output

none

* Function 2

Hidden cursor

This function removes cursor from the screen.

Input

AX = 2

Output

none

* Function 3

Get mouse position & button status

This function reports current mouse position & button status.

button down a bit is 1

button up a bit is 0

Input

AX = 3

Output

BX = button status

CX = horizontal cursor position

DX = vertical cursor position

* Function 4

Set mouse cursor position

This function sets cursor to a specified position on the screen.

Input

AX =4

CX = new horizontal cursor position

DX = new vertical cursor position

Output

none

* Function 5

Get button press information

This function reports

1. Current button status.

2. A count of button press since the last call to function 5

or function 0.

3. Horizontal and vertical position of the cursor, the button

was pressed last time.

Input

AX = 5

BX = button  ;  BX=0 check left button

;  BX=1 check right button

;  BX=2 check middle button

Output

AX = button status; bit 0 left button, bit 1 right button,

bit 2 middle button.

BX = number of button presses

CX = horizontal cursor position at last press

DX = vertical cursor position at last press

* Function 6

Get button release information

This function reports

1. Current button status.

2. A count of button release since the last call to function

6 or function 0.

3. Horizontal and vertical position of the cursor, the button

was released last time.

Input

AX = 5

BX = button  ; BX=0 check left button

;  BX=1 check right button

;  BX=2 check middle button

Output

AX = button status; bit 0 left button, bit 1 right button,

bit 2 middle button.

BX = number of button releases

CX = horizontal cursor position at last released

DX = vertical cursor position at last released

* Function 7

Set MIN & MAX X position

This function is used to set mouse horizontal moving range.

Input

AX = 7

CX = new minimum horizontal cursor position

DX = new maximum horizontal cursor position

Output

none

* Function 8

Set MIN & MAX Y position

This function is used to set mouse vertical moving range.

Input

AX = 8

CX = new minimum vertical cursor position

DX = new maximum vertical cursor position

Output

none

* Function 9

Define graphics cursor block

This function determines what the shape and color of the cursor

will be when it is in graphics mode, and it identifies the center

of the cursor.  The cursor block is a 16 X 16 bit pattern.  The

cursor hot spot values must be within the range from -16 to 16.

They define one pixel within ( or outside) the cursor block.

Input

AX = 9

BX = horizontal cursor hot spot

CX = vertical cursor hot spot

DX = pointer to screen and cursor mask

Output

none

Note: If you use large mode to call this function register ES

must be set to the segment value of the screen and cursor mask

array.

* Function 10

Define text cursor

This function selects the software or hardware cursor.

The value of the parameter BX selects the cursor type value is 0,

software text cursor is selected.

CX specify the screen mask

DX specify the cursor mask

value is 1, hardware cursor is selected.

CX contain the first scan line

DX contain the last scan line

Input

AX = 10

BX = select cursor

CX = screen mask value/scan line start

DX = cursor mask value/scan line stop

Output

none

* Function 11

Read mouse motion counters

This function returns the horizontal and vertical step count,

which is the distance the mouse has moved in 1/200 inch

increments, since the last call to this function.  The step count

is always within the range from -32768 to 32767.  A positive

horizontal count is a motion to the right while a positive

vertical count is a motion to the bottom of the screen.  The step

count is set to 0 after the call is completed.

Input

AX = 11

Output

CX = horizontal count

DX = vertical count

* Function 12

Define event handler

This function sets the call mask and subroutine address for the

mouse software interrupts.  The software interrupts stop

execution of your program and call the specified subroutine

whenever on or more of the conditions defined by the call mask

occur.  The call mask is a single integer value which defines the

conditions to cause an interrupt.  Each bit in the call mask

corresponds to a specific condition:

Mask Bit        Condition

0            change cursor position

1            press left button

2            release left button

3            press right button

4            release right button

5            press middle button

6            release middle button

7-15         non used

Input

AX = 12

CX = call mask

ES:DX = address of subroutine

Output

* Function 13

Light pen emulation mode on

This function enables light pen emulation by the mouse.  when the

mouse emulates the light pen, calls to the pen function will

return the cursor position at the last pen down which is

controlled by the mouse buttons.  The pen is down when any button

is down.  The pen is off the screen when both buttons are up.

Input

AX =13

Output

none

* Function 14

Light pen emulation mode off

This function disables the light pen emulation mode.

Input

AX = 14

Output

none

* Function 15

Set mouse motion/pixel ratio

This function sets the mouse motion to screen pixel ratio.  The

horizontal and vertical ratios specify the amount of mouse motion

at 8 pixels with the values falling within the range from 1 to

32767.  The default values are 8 steps to 8 pixels horizontally

and 16 steps to 8 pixels vertically.  This is equivalent to 3.2

inches of horizontal mouse movement and 2.0 inches of vertical

mouse movement.

Input

AX = 15

CX = horizontal step to pixel ratio

DX = vertical step to pixel ratio

Output

none

* Function 16

Disable cursor display in special range

This function allows the user to define an area on the screen

within which the mouse will be turned off.  Performing the Show

Cursor Function - Function 1 - resets the region and turns the

mouse back on.  This function is used to guard a portion of the

screen which your program is abort to update.

Input

AX = 16

CX = left margin

DX = top margin

SI = right margin

DI = bottom margin

Output

none

* Function 19

Defined double speed threshold

This command define double-speed threshold

Input

AX = 19

DX = threshold speed in mouse motions/second

Output

none

* Function 20

Swap event handler entery location

This command sets new values for the call mask and event handler routine

address for mouse hardware interrupts and return the values that were pre-

viously specified.

Input

AX = 20

CX = new call mask

DX = new point to event handler routine

Output

CX = old call mask

DX = old pointer to event handler routine

* Function 21

Get mouse driver state storage size

This command gets mouse driver state storage size. It is used with function

22 and 23 when you want to interrupt a program that is using the mouse and

execute another that also using the mouse.

Input

AX = 21

Output

BX = buffer size required for mouse driver state

* Function 22

Save mouse driver state

This command saves mouse driver state.It is used with function 21 and 23.

Input

AX = 22

DX = pointer to the buffer

Output

none

* Function 23

Restore mouse driver state

This command restores the last mouse driver state saved by function 22. It

is used with function 21 and 22.

Input

AX = 23

DX = pointer to the buffer

Output

none

* Function 29

Set CRT page number

This command specifies the CRT page on which the mouse cursor will be

displayed. For more information on the number of CRT pages available in each

display mode ,see documentation that came with the graphics adapter.

Input

AX = 29

BX = CRT page for mouse cursor display

Output

none

* Function 30

Read CRT page number

This command returns the number of the CRT page on which the mouse cursor

is displayed.

Input

AX = 30

Output

BX = CRT page number of current cursor display